import os

import SimpleITK as sitk

import numpy as np

from skimage import measure

from stl import mesh

# Step 1: Load DICOM files from a directory

def load_dicom_series(directory):

"""Load a DICOM series and return the 3D volume and spacing."""

try:

reader = sitk.ImageSeriesReader()

dicom_files = reader.GetGDCMSeriesFileNames(directory)

if not dicom_files:

raise ValueError("No DICOM files found in the specified directory.")

reader.SetFileNames(dicom_files)

image = reader.Execute()

volume = sitk.GetArrayFromImage(image)  # Convert to numpy array

spacing = image.GetSpacing()[::-1]  # Flip spacing order to match numpy array (z, y, x)

return volume, spacing

except Exception as e:

print(f"Error loading DICOM series: {e}")

raise

# Step 2: Segment the Region of Interest (ROI)

def segment_volume(volume, lower_threshold, upper_threshold):

"""Apply thresholding to create a binary mask for the region of interest."""

print(f"Segmenting with threshold {lower_threshold} - {upper_threshold}")

segmented = np.where((volume > lower_threshold) & (volume < upper_threshold), 1, 0)

return segmented

# Step 3: Generate Mesh

def generate_mesh(segmented_volume, spacing):

"""Generate a triangular mesh from the segmented volume using marching cubes."""

print("Generating mesh using marching cubes...")

verts, faces, _, _ = measure.marching_cubes(segmented_volume, level=0, spacing=spacing)

print(f"Generated {len(verts)} vertices and {len(faces)} faces.")

return verts, faces

# Step 4: Save Mesh as STL File

def save_stl(verts, faces, output_file):

"""Save the triangular mesh as an STL file."""

try:

print(f"Saving STL file to {output_file}...")

stl_mesh = mesh.Mesh(np.zeros(faces.shape[0], dtype=mesh.Mesh.dtype))

for i, f in enumerate(faces):

for j in range(3):

stl_mesh.vectors[i][j] = verts[f[j], :]

stl_mesh.save(output_file)

print(f"STL file saved successfully to: {output_file}")

except Exception as e:

print(f"Error saving STL file: {e}")

raise

# Main Code

# Specify the DICOM folder and output file

dicom_dir = "/path/to/dicom/folder"  # Change this to your DICOM folder path

output_dir = "/content/stl_output"   # Directory to save the STL file

output_stl = os.path.join(output_dir, "output.stl")

# Ensure the output directory exists

os.makedirs(output_dir, exist_ok=True)

try:

# Load DICOM series

print("Loading DICOM series...")

volume, spacing = load_dicom_series(dicom_dir)

print(f"Volume shape: {volume.shape}, Spacing: {spacing}")

# Segment the volume (adjust thresholds as needed)

lower_threshold = 100  # Adjust this based on your data

upper_threshold = 300  # Adjust this based on your data

print("Segmenting the volume...")

segmented = segment_volume(volume, lower_threshold, upper_threshold)

# Generate the mesh

print("Generating the mesh...")

vertices, faces = generate_mesh(segmented, spacing)

# Save the STL file

print("Saving the STL file...")

save_stl(vertices, faces, output_stl)

print(f"STL file saved to: {output_stl}")

except Exception as e:

print(f"An error occurred: {e}")